HARA analysis for Robot One

Date: 10-10-2025

Version: 0.3

Status: validated

Author: D. Gijsbers

# Version control

| Version | Date | Description of changes |
| --- | --- | --- |
| 0.1 | 25-09-2025 | Added: Introduction, What is HARA, Why perform a HARA |
| 0.2 | 26-09-2025 | Added: system definition |
| 0.3 | 29-09-2025 | Added: HARA table, conclusion |
| 0.4 | 10-10-2025 | Changed: Feedback on HARA table |

Table of contents

Version control	2

Introduction	4

What is HARA?	5

Key elements of HARA	5

Why perform a HARA analysis?	7

System definition	8

System boundaries	8

Core functions	8

Operational environment	9

Interface with humans	9

Assumptions and limitations	9

HARA table	10

Conclusion	12

# Introduction

As part of the graduation project in ICT & Robotics, a comprehensive safety framework has been developed for Robot One, the autonomous agricultural robot designed by Pixelfarming Robotics.

One of the elements of this framework is the Hazard Analysis and Rick Assessment (HARA). The HARA analysis focus on systematically identify the potential hazards, evaluate their risks and define safety requirements. These requirements form the basis of the safety architecture, making sure that safety is not an afterthought but a fundamental design driver.

This report provides an exploration of the HARA analysis for the Robot One. It covers the theoretical information of the HARA, the methodology, the results of the risk assessment and how these finding chance the overall system and software design.

# What is HARA?

Hazard Analysis and Risk Assessment (HARA) is a method from functional safety engineering, originally developed for the automotive standard ISO 26262 (ISO, 2018) intended to make safety-related risks explicit, mensurable and manageable. HARA provides a structured way to change the vague concerns about “What can go wrong” into concrete hazard definitions, risk evaluations and design requirements.

## Key elements of HARA

Hazard identification

The first step is to systematically mention hazards: Deviations from the intended system behaviour that could cause harm (to people, the environment, or property). In practice, this can be analysed with methods like HAZOP,  or scenario-based brainstorming.

Risk evaluation

For each hazard, three areas are considered:

- Severity (S): the seriousness of consequences if the hazard occurs — from minor injury to life-threatening or fatal outcomes.

- Exposure (E): how often or how likely the operational situation (in which the hazard may materialize) occurs. In other words, how frequently the system is in that “dangerous condition.”

- Controllability (C): the degree to which a human or system can intervene to avoid or mitigate harm once the hazard begins. It reflects how much control remains to prevent or reduce damage.

These three factors give a qualitative or semi-qualitative risk rating for each hazard. The result can be a general “risk level” or, in the automotive field, an ASIL (Automotive Safety Integrity Level).

Risk classification & ASIL Assignment

Once S,E and C are determined, they are combined (normally via a lookup table or predefined rules in ISO 26262) (ISO, 2018) to map the hazards to risk classes. ASIL A, B, C, D or QM (Quality Management) for hazards whose risk is considered sufficiently low to be handled by standard quality processes. Higher ASIL number demand stronger safety measures, more validation, redundancy and diagnostic coverage.

Definition of Safety Goals & Measures

Once hazard risk levels are known, the next step is to define safety goals (top-level requirements that must always be met) and more detailed safety requirements to mitigate or control hazards. These requirements guide the architectural, hardware, software, and operational design of the system.

Traceability & Validation

A key strength of HARA is that it provides traceability: every safety measure, design choice, or validation test can be linked back to a specific hazard and its risk evaluation. This traceability is important not only for careful design but also for certification, audits, and safety assurance.

# Why perform a HARA analysis?

A HARA analysis is essential when developing autonomous and safety-critical systems like Robot One. HARA shows what hazards are and how to classify them, but its real value is that it makes safety demonstrable, traceable, and systematic.

Avoiding Arbitrary Measures

Without a structured risk assessment, safety measures may be chosen randomly or based only on intuition. HARA makes sure that every safety mechanism (like redundant sensors, emergency stops, or watchdog timers) is directly connected to a specific hazard. This gives clear justification for engineering choices.

Traceability from Hazard to Design

In ISO 26262 and ISO 25119, traceability is a core requirement. HARA provides this by mapping hazards to risks to safety goals to technical measures. This creates a transparent chain of reasoning that auditors, certification bodies, or stakeholders can review.

Prioritization of the risks

Not all hazards are equally dangerous. Some happen often but have minor effects, while others are rare but could be fatal. HARA helps teams focus resources where they matter most — on high-risk hazards with high severity, frequent exposure, and low controllability.

Compliance with international standards

For automotive systems, ISO 26262 makes HARA mandatory in the first phase (Concept phase). For agricultural machinery, ISO 25119 also requires systematic hazard and risk assessment to define safety requirements. Compliance with these standards makes sure not only safety but also legal and regulatory acceptance of the product. (ISO, 2018) (ISO, 2018a) (Official Journal of the European Union, 2023)

Transparency and reproducibility

By documenting hazards, risk levels and iterations, HARA provides transparency. It ensures safety decisions are not hidden in assumptions but are made explicit and reproducible. This allows future engineers to understand, verigy and improve the systems safety concept.

Summary

In summary, performing a HARA is not just about hazard identification but is about create a safety argument that proves why the system architecture includes the measures it does. This makes HARA an important part of both engineering practice and compliance.

# System definition

A clear system definition is the foundation for any Hazard Analysis and Risk Assessment (HARA). It sets the boundaries of the system, identifies its functions, and describes the environment in which it operates. For Robot One, the system definition ensures that all hazards are analysed within a consistent and realistic context.

## System boundaries

Robot One is an autonomous farm robot developed by Pixelfarming Robotics. Its main purpose is to support precision farming by performing repetitive field tasks with high accuracy and little human intervention. Unlike traditional tractors, Robot One is lightweight, electric, and fully autonomous. This reduces soil compaction and emissions but also brings new risks related to software, perception, and autonomy. (Leveson, 2012)

The boundaries of the system include:

- Onboard subsystems such as navigation, perception sensors, power supply, actuators, tool interfaces, and communication units.

- Human-machine interfaces (HMI) including emergency stop buttons, remote operator dashboards, and override controls.

- External systems like GNSS/RTK base stations, cloud-based control software, and wireless communication links.

## Core functions

Robot One integrates several core functions, each of which is safety-relevant in different ways:

- Autonomous Driving and Navigation: The robot follows pre-set field routes using GNSS/RTK positioning together with sensors for row-following. It must detect and avoid unexpected obstacles while staying precisely aligned with crop rows.

- Perception and obstacle detection: The robot has multiple camera’s to detect humans, animals and objects. Sensor fusion will improve reliability, but failures can directly increase the risk of collisions.

- Agricultural tool operation: Robot One can be equipped with tools such as a mechanical weeder and a laser. These tools are actuated autonomously and require safeguards to prevent accidental activation.

- Communication and remote control: A wireless link connects the Robot One with an operator dashboard. The operator can monitor operations, update missions and trigger an e-stop. The communication between these must be secure and reliable to prevent unauthorized access.

- Emergency stop and fail-safe mechanisms: More than one E-stop are available both on the robot and remotely. When the E-stop is triggered, the system must transition into the safe state (Stopping all motion and deactivating tools).

## Operational environment

Robot One is designed to work in open farm fields, which brings unique challenges compared to structured road environments:

- Presence of Humans and Animals: Farm workers may enter the field unexpectedly. Children or passers-by may come out of curiosity. Wildlife or domestic animals can also cross the robot’s path. (ISO, 2016)

- Physical Obstacles: Fields may contain fences, poles, stones, irrigation lines, or tools and machines left behind. These must be detected and avoided.

- Terrain Variability: Farm terrain is often uneven. Soft soil, mud, ruts, slopes, and bumps can cause slipping, skidding, or even tipping over.

- Weather Conditions: Rain, fog, dust, or snow can reduce sensor performance, visibility, and traction. The robot must remain safe under these conditions. (Kalra, 2016)

- Electromagnetic and Communication Interference: GNSS signals can be blocked by trees or buildings, and wireless communication can be disturbed in rural areas. Strong fallback strategies are needed.

## Interface with humans

The interaction between Robot One and humans plays a big role in define the hazards:

- Operators needs to be able to monitor the robot status and intervene when needing.

- Safety signage, HMI warnings and training reduce the risk of not using it correctly.

- Maintenance personnel must have access to the safe service procedures, including lock-out/tag-out protocols for tool safety.

## Assumptions and limitations

The system definition is based on the assumption that Robot One will primarily operate in controlled farm environments rather than public roads. However, since farms are not completely closed to human presence hazards involving bystanders and animals remain highly relevant.

# HARA table

The risk parameters S, E, C follow the method described in ISO 26262 (ISO, 2018) and ISO 25119 (ISO, 2018a)

| Function/ Domain | Hazard | Scenario Example | S | E | C | Risk Level | How to Solve |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Autonomous Driving | Collision with human/animal | Worker enters field unexpectedly | 3 | 2 | 3 | High | Sensor fusion, emergency stop, safety zones |
|  | Collision with object | Robot fails to detect pole/fence | 2 | 3 | 2 | Medium | Redundant perception sensors |
|  | Driving outside field boundaries | GPS signal lost | 2 | 2 | 3 | High | Geo-fencing, localization redundancy |
|  | Incorrect localization | Multipath GNSS error → mispositioning | 2 | 2 | 2 | Medium | GNSS + IMU + vision sensor fusion |
| Tool control | Unintended tool activation | Software fault triggers weeding laser | 3 | 2 | 3 | High | State machine, hardware safety relay |
|  | Tool active during servicing | Maintenance performed while tool is powered | 3 | 1 | 3 | High | Lock-out/tag-out, interlocks |
| Navigation / Mobility | Skidding or rollover | Robot loses balance on wet soil | 2 | 2 | 2 | Medium | IMU, terrain monitoring, stability control |
|  | Soil collapse / sinking | Robot sinks in soft ground | 2 | 2 | 2 | Medium | Ground pressure monitoring, low weight design |
| Perception | Missed obstacle | Camera blocked by mud | 2 | 3 | 3 | High | Cleaning system, radar fallback |
|  | Sensor calibration drift | IMU becomes inaccurate | 2 | 2 | 2 | Medium | Calibration, self-diagnostics |
|  | Vegetation interference | Crops block camera/lidar | 2 | 3 | 2 | Medium | Sensor placement optimization, AI filtering |
| Communication | Loss of communication | Remote operator loses link | 2 | 3 | 2 | Medium | Fail-safe stop mode |
|  | Unauthorized control takeover | Attacker sends malicious commands | 3 | 1 | 3 | High | Encryption, authentication, intrusion detection |
|  | Denial of Service (DoS) | Network flooded, robot unresponsive | 2 | 2 | 3 | Medium | Redundant channels, monitoring |
| Power / Battery | Sudden power failure | Battery disconnects mid-operation | 2 | 2 | 2 | Medium | Power redundancy, brownout detection |
|  | Battery fire (thermal runaway) | Overheating during charging | 3 | 1 | 3 | High | Temperature monitoring, certified battery system |
| Hardware / Actuators | Wheel actuator stuck | Motor blocked → robot veers | 2 | 2 | 2 | Medium | Motor feedback loops, safe shutdown |
|  | Electrical short circuit | Cabling damage causes spark | 2 | 1 | 3 | Medium | Fusing, isolation, protective housings |
| Emergency systems | Emergency stop failure | E-stop pressed but robot does not stop | 3 | 1 | 3 | High | Dual-channel circuits, watchdog monitoring |
|  | Unsafe recovery after fault | Robot restarts with actuators engaged | 3 | 1 | 3 | High | Defined safe state, reset logic |
| Environmental | Extreme weather | Fog/rain reduces visibility | 2 | 3 | 3 | High | Multi-modal sensing (thermal, radar) |
|  | Dust/mud contamination | Sensors clogged | 2 | 3 | 2 | Medium | Sealing, cleaning routines |
| Human interaction | False sense of safety | Operator trusts robot blindly | 2 | 2 | 2 | Medium | Training, HMI warnings |
|  | Misuse of manual override | Operator disables safety features | 3 | 1 | 3 | Medium | Restricted access, override logging |

# Conclusion

The extended HARA analysis shows that the risks associated with Robot One are diverse and span across navigation, perception, tool control, communication, power and emergency systems.

By identifying and evaluating these hazards step by step, the analysis clearly supports the chosen safety design, which includes redundancy, secure communication, fail-safe mechanisms, and strong emergency systems.

The outcome of this HARA is a clear and traceable safety concept that meets regulations, showing that safety is not an add-on but a core design principle of Robot One.

# Bibliografie

ISO. (2016). Robots and robotic devices – Collaborative robots. Opgehaald van ISO/TS 15066:2016: https://www.iso.org/standard/62996.html

ISO. (2018). Road vehicles – Functional safety – ISO 26262. Opgehaald van ISO.

ISO. (2018a). “Agricultural machinery safety is structured through a lifecycle process. Opgehaald van ISO 25119: https://www.iso.org/standard/69025.html?

Kalra, N. &. (2016). Driving to safety: How many miles of driving would it take to demonstrate autonomous vehicle reliability? Opgehaald van https://doi.org/10.7249/RR1478

Leveson, N. G. (2012). Engineering a Safer World: Systems Thinking Applied to Safety. MIT Press Direct.

Official Journal of the European Union. (2023). Regulation (EU) 2023/1230 of the European Parliament and of the Council of 14 June 2023 on machinery. Opgehaald van legal-content: https://eur-lex.europa.eu/legal-content/EN/TXT/?uri=CELEX%3A32023R1230

Pixelfarming Robotics. (2025). Robot One System Documentation. Opgehaald van Pixelfarming Robotics.