Safety in autonomous robotics and relevant standards

Date: 22-09-2025

Version: 0.3

Status: Review

Author: D. Gijsbers

# Version Control

| Version | Date | Description of changes |
| --- | --- | --- |
| 0.1 | 15-09-2025 | Added: Introduction, Safety in autonomous robotics |
| 0.2 | 16-09-2025 | Added: Relevant safety standards |
| 0.3 | 22-09-2025 | Added: Best practices, Discussion, Conclusion, Sources |

Table of Contents

Version Control	2

Abstract	4

Introduction	5

Safety in autonomous robotics	6

Introduction safety	6

Fundamental safety principles	6

Safety issues	7

Lessons from related domains	7

Relevance	8

Relevant safety standards	9

Introduction safety standards	9

ISO 25119 Agricultural machinery functional safety	9

ISO 26262 Functional safety in automotive	9

ISO 13482 Safety for personal care robots	10

ISO 10218 & ISO 15066 Industrial and collaborative robots	10

European machinery regulation (EU 2023/1230)	10

Conclusion relevant safety standards	11

Best practices	12

Autonomous vehicles	12

Industrial and collaborative robotics	12

Consumer robotics	13

Agricultural Robotics	13

Naïo Technologies	13

AgXeed	14

Relevance for Robot One	15

Key Lessons Across Domains	15

Conclusion best practices	17

Discussion	17

Conclusion	18

Sources	19

# Abstract

Increasing the use of autonomous robots in agriculture environments offers promising opportunities for efficiency and sustainability but also raises safety concerns. Unlike industrial robots, agricultural robots operate in dynamic environments where interactions with humans, animals, and natural changes happen. This study reviews the scientific and industrial literature on safety in autonomous robotics, check relevant international standards and regulations, and identifies best practices from related domains such as automotive robotics and industrial robotics. The results feature the importance of fail-safe mechanisms, redundancy, structured testing, and consent with the ISO standards such as ISO 25119 and ISO 26262. These insights will serve as the base for designing a safety protocol for Robot One, an agricultural robot developed by Pixelfarming Robotics, which is already deployed on European farms.

# Introduction

The development of autonomous robots in agriculture represents a growing technological trend that promises to reduce labour, increase efficiency, and promote sustainable farming practices. Robot One, designed by Pixelfarming Robotics, is such a robot. Importantly, Robot One is already commercially available and has been sold to farmers across Europe, with around five units currently in use in real agricultural environments.

This commercial deployment increases the urgency of safety concerns. Unlike research prototypes, Robot One is actively operating in dynamic, real-world contexts, where unsafe behaviour could impact farmers, workers, and the reputation of Pixelfarming Robotics. Failures are no longer hypothetical risks but potential real-world incidents. Therefore, this research not only aims to support future development but also addresses the immediate need for a safety protocol that ensures safe operations of the robots already in the field.

# Safety in autonomous robotics

## Introduction safety

Safety is one of the most critical aspects in the development and deployment of autonomous robots (Kalra, 2016). Unlike normal automated machines, who operate in an industrial environment, Robot One must deal with difficult, dynamic, and unpredictable real-world conditions. Safety is therefore not only a matter of technical design but also of managing compliance, user trust, and ethical responsibility. For agricultural robots such as Robot One, safety has an even greater significance because these machines interact directly with farmers, workers, animals, and crops in real-world environments.

## Fundamental safety principles

From the robotics and autonomous systems literature, several core principles show up as the base of safety:

- Fail-safe design: The system must enter a safe state when a failure occurs. In most cases, this means halting operations until the issue is resolved.

- Redundancy: Critical components, such as sensors or communication systems, must be duplicated or backed up to avoid single points of failure.

- Fault tolerance: Beyond redundancy, systems must be able to continue operating safely in degraded modes, even when some functions are compromised.

- Speed and separation monitoring: Originating from collaborative robotics, this principle requires that the robot dynamically adjust its speed and motion based on the closeness of humans and obstacles.

These principles form the backbone of modern safety protocols and are often embedded in international standards such as ISO 26262 (automotive) (ISO, 2018b) and ISO 15066 (collaborative robotics) (ISO, 2016).

## Safety issues

Ensuring safety in autonomous systems is challenging due to several factors:

- Sensor reliability: Sensors act as the robot’s eyes and ears. Harsh conditions such as dust, rain, or poor lighting can reduce their accuracy, leading to wrong decisions. (Blackmore, 2019)

- Uncertainty in perception: Robots use AI models to interpret their surroundings. Mistakes in detection, for example confusing a bush with a person, can cause unsafe actions. (Kalra, 2016)

- Complex environments: Unlike controlled factory settings, roads and fields are open and unpredictable, with people, animals, and moving machines creating constant changes. (Blackmore, 2019)

- Software complexity: As systems grow with many interacting modules, the risk of unexpected behaviours, unusual situations, or hidden bugs that affect safety also increases. (Leveson, 2012)

- Ethical considerations: Robots must also make value-based decisions, such as when to stop and how to balance risks. For instance, stopping too abruptly could harm crops, while stopping too late could endanger a person. (E, 2016)

## Lessons from related domains

Safety in robotics often uses other lessons from other domains:

- Automotive: Self-driving cars use redundancy, fail-safe braking, and scenario-based validation. These methods are directly transferable to field robots. (Kalra, 2016) (ISO, 2018b)

- Industrial robotics: Collaborative robots (cobots) have pioneered methods such as protective separation distance and speed adaptation (ISO, 2016) (ISO, 2011).

- Medical robotics: Surgical robots needs a lot of validation and documentation, showing how important of traceability and accountability in safety-critical systems are. (E, 2016)

## Relevance

For agricultural robots such as Robot One, the above principles and challenges are requirements. Motion detection must not only recognize people, but also be able to make a separation between workers, animals, and plants. A fail-safe design must ensure that the robot can stop safely on uneven terrain without causing damage. Redundancy is important when sensors are exposed to dust, mud, or glare. Scenario-based tests should include agricultural conditions, such as narrow rows, wet fields, or unexpected encounters with workers.

In short, safety in autonomous robots is not a single feature or algorithm, but a system-level approach that combines principles, standards, and best practices. For Robot One, this means that safety must be built into not only the design of hardware and software, but also into validation, documentation, and long-term compliance.

# Relevant safety standards

## Introduction safety standards

Safety standards give structured guidelines to make sure complex systems work reliably and reduce risks for people, animals, and the environment. For autonomous farm robots like Robot One, following international standards is important not only for technical safety but also for meeting regulations and gaining user trust. This chapter looks at the most relevant standards and regulations, and explains how they apply to Robot One.

## ISO 25119 Agricultural machinery functional safety

ISO 25119 (ISO, 2018a) specifies safety requirements for the design and development of control systems in agricultural machinery. It follows a lifecycle approach, its similar to ISO 26262 in automotive and is custom made for the agricultural field. The standard includes:

- Risk assessment and hazard analysis for agricultural environments

- Categorization of safety-related parts of control systems into performance levels.

- Design principles for ensuring that failures in electronic control systems do not lead to unsafe machine behaviour.

For the Robot one, ISO 25119 has the most direct guidance. The standard would structure the way the engineers at Pixelfarming Robotics analyse hazards such as unexpected presence of humans, sensor failures or unexpected robot movement on slopes. The protocol can define safety integrity levels for critical systems (emergency stop, obstacle detection) and the standard provide testing requirements for validation.

## ISO 26262 Functional safety in automotive

ISO 26262 (ISO, 2018b)defines a complete framework making sure for the safety of electrical and electronic systems in road vehicles. It introduces the concept of Automotive Safety Integrity Levels (ASILs), which classify risks based on severity, exposure, and controllability. The standard emphasizes:

- Hazard analysis and risk assessment (HARA).

- Safety lifecycle management.

- Verification and validation methods.

Although ISO 26262 was first made for cars, many of its ideas can also be used in farming. For example, you can study risks and classify them: hitting a worker with a machine would be seen as very serious and likely to happen, so it needs strong safety measures. The idea of ASILs could also be used to create similar safety levels for agriculture.

## ISO 13482 Safety for personal care robots

ISO 13482 (ISO, 2014)defines requirements for robots that are designed to interact with humans, such as service and personal care robots. The focus is on safe interaction, limiting forces and maintaining a safe distance.

Even though Robot One is not a service robot, working near people in the field makes these principles important. For example, limits on speed and force can be used when the robot is close to farm workers, and keeping safe distances can make sure the robot slows down or stops when people come near.

## ISO 10218 & ISO 15066 Industrial and collaborative robots

ISO 10218 (ISO, 2011)specifies safety requirement for industrial robots and ISO 15066 (ISO, 2016)expands these requirements for collaborative robots that share a workspace with humans. These standards highlights:

- Protective measures such as emergency stops and safety-rated monitored stops.

- Speed and separation monitoring.

- Power and force limiting to prevent injuries.

Although originally made for industrial settings, the idea of speed and separation monitoring is also relevant here. For example, if a worker comes into the robot’s working area, Robot One should slow down or stop. Power and force limits can also be applied to make sure that if contact happens, the risk of injury is kept as low as possible.

## European machinery regulation (EU 2023/1230)

The EU machinery regulation (Official Journal of the European Union, 2023) replaces the former machinery directive and introduces more strict rules for AI-based and autonomous machinery. The rules state that the next things need to be done:

- Documentation of risk assessments for all intelligent functions.

- Transparency in decision-making for autonomous systems.

- Stronger emphasis on cybersecurity as part of safety.

Since Robot One is already sold in Europe, following this regulation is required. Because of the new focus on AI, functions like motion detection and autonomous navigation must be clear and traceable. Engineers must show not only that the robot is safe, but also how its decisions are made and checked.

## Conclusion relevant safety standards

Safety standards form the foundation for managing risks and ensuring robots behave safely. For Robot One, ISO 25119 is the most important standard because it applies directly to agricultural machines. In addition, ISO 26262 (for risk assessment) and ISO 15066 (for human-robot interaction) offer useful extra guidance. The EU Machinery Regulation introduces a binding legal framework that will affect not only future models but also existing Robot One units already in the field.

Together, these standards show that safety should be treated as a process that lasts throughout the robot’s life, combining design, testing, validation, and documentation into a complete system.

# Best practices

Developing a safety protocol for Robot One can benefit from looking at best practices in other fields where autonomous systems are already used (Blackmore, 2019). Lessons from automotive, industrial robotics, consumer robotics and existing agricultural robots demonstrate how safety is embedded into both technical design and organizational processes. By studying these fields, we can find useful principles and adjust them to fit the specific challenges of agriculture.

## Autonomous vehicles

The automotive industry is one of the leading fields in autonomous safety. Companies such as Waymo, Tesla and Volvo have invested a lot in developing and validating safety frameworks.

- Scenario-based testing is often used, where vehicles are tested in thousands of virtual and real situations, including rare and unusual cases. (Kalra, 2016)

- Sensor fusion combines data from LiDAR, radar, and cameras to improve reliability. Redundancy makes sure that if one sensor fails, the others can take over. (ISO, 2018b)

- Safety lifecycle processes based on ISO 26262 make sure that risks are identified and reduced step by step during design, implementation, and validation. (ISO, 2018b)

Agricultural robots face similar unexpected situations as cars, sudden appearance of obstacles, variable terrain and mixed traffic with humans and other machines. The principles of redundancy, risk classification, and scenario-based testing can also be adapted for Robot One.

## Industrial and collaborative robotics

Industrial robots traditionally operated in isolated environments, but the rise of collaborative robots (cobots) has driven new safety innovations.

- Speed and separation monitoring (SSM): the robot slows down or stops when a person comes close to its working area. (ISO, 2016)

- Power and force limiting (PFL): cobots are built so that contact with humans does not cause injury.

- Safety-rated monitored stops: Robots immediately pause motion if an unexpected event is detected.

Although agricultural robots do not share workspaces as closely as cobots, the concept of protective separation distances and force limitation is highly relevant. For example, Robot One should slow down when workers or other robots are nearby and make sure that emergency stops can always prevent collisions.

## Consumer robotics

Consumer robots demonstrate how safety protocols are implemented at smaller scales but with strict regulatory oversight.

Robot lawnmowers (Husqvarna Automower) must comply with IEC 60335-2-107 (Household and similar electrical appliances – Safety – Part 2-107 (IEC, 2017): Particular requirements for robotic battery powered electrical lawnmowers.), which prescribes features such as:

- Tilt and lift sensors that immediately stop blades when the mower is picked up.

- Obstacle detection that redirects motion on collision.

- Boundary wires or geofencing to restrict the working area.

- Emergency stop buttons that instantly shut down operations.

Robot vacuum cleaners (iRobot Roomba) use:

- Cliff sensors to prevent falls down stairs.

- Soft bumpers and collision sensors to minimize impact.

- Automatic shutdown when lifted or blocked.

Although they are less dangerous, consumer robots show that formal safety rules and certifications are required even for small machines. For Robot One, which is larger and more powerful, using structured and redundant safety systems is even more important.

## Agricultural Robotics

In recent years, several companies have built and sold autonomous robots for farming. Their size, purpose, and level of autonomy are different, but they all face the same main challenge: ensuring safety in changing farm environments. Two well-known examples are Naïo Technologies and AgXeed.

### Naïo Technologies

Naïo Technologies (Naïo Technologies, 2025), a French company, produces a range of agricultural robots primarily focused on weeding and crop management. Examples include:

- Oz: a small robot for market gardens and small farms. It weeds between crop rows on its own and can carry small loads.

- Dino: a larger robot for weeding vegetable fields. It uses GPS and computer vision to navigate.

- Ted: a vineyard robot that works between vines, with functions like mechanical weeding and row maintenance.

Naïo’s robots typically integrate the following safety features:

- Geofencing and GPS boundaries to limit the operating area.

- Obstacle detection sensors (ultrasonic and LiDAR) to identify humans, animals, or machinery.

- Emergency stop buttons mounted on the robot, allowing workers to shut it down instantly.

- Remote supervision via a tablet interface, enabling operators to intervene when necessary.

### AgXeed

AgXeed (AgXeed, 2025), a company from the Netherlands, develops large autonomous tractors for arable farming. Their main product is the AgBot:

- AgBot 5.115T2: a mid-sized tracked vehicle used for soil preparation, seeding, and crop care. It runs on a diesel-electric drive, which combines fuel efficiency with reliable power. The AgBot can work with many standard farm tools, making it flexible for different tasks on the field. It is designed to operate autonomously, reducing the need for manual labour while increasing consistency in farming operations.

AgXeed Maintains:

- Advanced geofencing and path planning, ensuring the machine stays within the designated working zone.

- Redundant positioning systems (RTK-GPS combined with inertial measurement) for accurate navigation.

- Collision avoidance systems based on LiDAR and camera fusion.

- Remote monitoring via cloud-based interfaces, enabling farmers to supervise multiple robots simultaneously.

### Relevance for Robot One

While Naïo Technologies builds smaller robots for tasks like weeding and vineyard work, AgXeed focuses on large-scale field operations similar to traditional tractors. Both companies use layered safety systems, such as geofencing, obstacle detection, emergency stops, and remote supervision.

For Robot One, which falls between these two extremes (smaller than AgBot but larger and more multifunctional than Naïo’s Oz) these examples show different ways to approach safety:

- From Naïo, Robot One can take over principles like working safely near people, frequent interaction with humans, and light emergency measures

- From AgXeed, Robot One can take over large-scale safety strategies, such as backup navigation systems, cloud monitoring, and strict geofencing.

These case studies show that even though agricultural robots differ in size and purpose, their safety strategies all follow a layered model: geofencing, obstacle detection, emergency stops, and remote monitoring.

## Key Lessons Across Domains

The review of best practices from fields like automotive, industrial robotics, consumer robotics, and agriculture shows clear lessons on how safety should be handled in autonomous systems. These lessons provide a foundation for understanding how safety can be organized and applied in different technological contexts.

Safety must be systematic and lifecycle-based

Safety cannot be added as an afterthought at the end of development. In both the automotive industry (ISO 26262) and agricultural machinery (ISO 25119), safety is seen as a lifecycle that starts with hazard analysis and continues through design, implementation, validation, and maintenance. A loose approach, where safety features are added separately without integration, is not enough (ISO, 2018a) (ISO, 2018b).

Redundancy and fault tolerance are essential

Best practices from the automotive and industrial robotics domains show that safety-critical systems cannot depend on a single component. Self-driving cars use several sensors together (LiDAR, radar, and cameras), while collaborative robots use overlapping sensors to track how close people are. Redundancy makes sure that if one part fails, the system can still work safely or shut down in a controlled way. (Leveson, 2012)

Scenario-based testing is the main standard for validation.

Testing a safety system only in normal situations is not enough. In automotive and robotics, scenario-based validation is widely used, where systems are tested with rare but critical cases such as sudden obstacles, sensor problems, or unexpected human behaviour. Simulation environments are often used to explore edge cases that are too risky or impractical to replicate in reality. (Kalra, 2016)

Human interaction requires specific safeguards

Fields like collaborative robotics (ISO/TS 15066) and consumer robotics (IEC 60335-2-107) highlight the need for clear mechanisms to protect humans. Common safeguards include speed and separation monitoring, force limits, and emergency stop functions. These measures ensure that systems stay predictable and controllable, even when people or animals enter the working area. (ISO, 2016) (IEC, 2017)

Regulatory compliance and certification are unavoidable

In all fields, regulations make sure that safety measures are not only effective but also testable and transparent. For example, consumer robots like robotic lawnmowers must follow IEC 60335-2-107 before they can be sold. Automotive systems are certified under ISO 26262, and industrial robots under ISO 10218. Following international standards and local rules is therefore required for market approval and public trust. (Official Journal of the European Union, 2023)

Safety protocols are layered, not single-feature solutions

No single measure can guarantee safety. Systems in all fields use a layered approach that combines preventive measures (like geofencing and safe distances), responsive measures (like emergency stops and obstacle avoidance), and fallback modes (like fail-safe or fail-operational states). This layered design makes the system resilient against both expected and unexpected failures. (E, 2016)

Summary

Taken together, these lessons show that safety does not come from single features, but from integrated systems, validated processes, and compliance with regulations. Experiences across different fields make clear that safety needs a structured approach that combines redundancy, scenario-based testing, human-focused safeguards, regulatory alignment, and layered defence strategies.

## Conclusion best practices

Best practices from other fields show that safety is never achieved with just one measure, but through layered protocols that combine fail-safes, redundancy, and testing frameworks. Consumer robots show small-scale, practical safety solutions, while the automotive and aerospace industries highlight the need for systematic risk assessment and scenario-based testing. For Robot One, combining these lessons offers a roadmap for building a strong safety protocol, aligned with standards, that works for current field use and future scaling.

# Discussion

The literature review shows that agricultural robots can benefit from a mix of agriculture-specific standards (ISO 25119) and principles from automotive and collaborative robotics. The challenge, however, is to adapt these standards from other fields to the unique conditions of agriculture. Unlike cars, agricultural robots usually work at lower speeds, but in environments that are more variable and less predictable.

For Robot One, the most practical approach is to use ISO 25119 as the baseline and add selected practices from ISO 26262 and ISO15066, adapted to agricultural situations. Scenario-based testing and sensor redundancy will be key for showing safety in practice. The safety protocol must also give direct value to the Robot One units already in use, so that current customers benefit from better safety measures while development continues.

# Conclusion

This literature study reviewed safety in autonomous robotics, focusing on principles, standards, and best practices that can guide the development of a structured safety protocol. The findings show that safety in autonomous systems is not achieved through single features, but through systematic, lifecycle-based approaches supported by redundancy, testing, and regulatory compliance.

Several key insights emerged:

- Safety principles like fail-safe design, redundancy, speed and separation monitoring, and diagnostic systems are found across all fields and form the foundation of reliable autonomous operation.

- Standards and regulations give an important framework for structuring safety. ISO 25119 is directly relevant for farm machines, while useful insights also come from ISO 26262 (automotive), ISO 15066 (collaborative robots). The new EU Machinery Regulation (2023/1230) also adds binding rules for autonomous systems in the European market.

- Best practices from other fields highlight the importance of scenario-based testing, layered safety systems, and human-centred safeguards. Examples from automotive, industrial, consumer, and agricultural robotics show that safety needs both technical solutions and formal protocols.

- The current fragmentation in agricultural robotics shows the gap this project aims to address: while individual safety features exist, they are rarely brought together into a full, standards-based framework.

The context of this research makes the issue urgent: Robot One is already being used on farms in Europe. This means safety is not just a future requirement but an immediate need. Farmers and workers must be able to trust that the robot will work reliably in unpredictable field conditions. At the same time, Pixelfarming Robotics must follow changing standards and regulations to stay competitive and avoid legal or reputational risks.

This study gives the theoretical and regulatory foundation for the next phases of the project. The identified principles, standards, and best practices will be used as input for the upcoming hazard and risk analysis and for designing a structured safety protocol tailored to Robot One. The goal is to move from separate safety features to an integrated and validated protocol that supports both current use and future scaling.

# Sources

AgXeed. (2025). AgBot 5.115T2. Retrieved from https://www.agxeed.com/our-solutions/agbot-5-115t2/

Blackmore. (2019). Robotics and automation in agriculture. Springer. Retrieved from https://doi.org/10.1007/978-3-030-16849-0

E, H. S. (2016). Physical human–robot interaction. Springer.

IEC. (2017).

ISO. (2011). Robots and robotic devices – Safety requirements for industrial robots – Part 1: Robots. Retrieved from ISO 10218-1:2011: https://www.iso.org/standard/51330.html

ISO. (2014). Robots interacting directly with humans require additional safeguards. Retrieved from ISO 13482: https://www.iso.org/standard/53820.html

ISO. (2016). Robots and robotic devices – Collaborative robots. Retrieved from ISO/TS 15066:2016: https://www.iso.org/standard/62996.html

ISO. (2018a). “Agricultural machinery safety is structured through a lifecycle process. Retrieved from ISO 25119: https://www.iso.org/standard/69025.html?

ISO. (2018b). Automotive safety integrity levels (ASILs) provide a useful model for risk classification. Retrieved from ISO 26262: https://www.iso.org/standard/68383.html

Kalra, N. &. (2016). Driving to safety: How many miles of driving would it take to demonstrate autonomous vehicle reliability? Retrieved from https://doi.org/10.7249/RR1478

Leveson, N. G. (2012). Engineering a Safer World: Systems Thinking Applied to Safety. MIT Press Direct.

Naïo Technologies. (2025). Augmented autonomy. Retrieved from Naïo Technologies: https://www.naio-technologies.com/en/augmented-autonomy/

Official Journal of the European Union. (2023). Regulation (EU) 2023/1230 of the European Parliament and of the Council of 14 June 2023 on machinery. Retrieved from legal-content: https://eur-lex.europa.eu/legal-content/EN/TXT/?uri=CELEX%3A32023R1230